|  | Industrial Processes Automation MSc in Electrical and Computer Engineering Scientific Area of Systems, Decision, and Control Winter Semester 2014/2015 |  | Group: ____ __-_____ __-_____ __-_____ __-_____ |
| --- | --- | --- | --- |

1st Laboratory Assignment

Alarm System for Intrusion Detection

This work aims the implementation of an intrusion detection alarm system, in a restricted space as a single room retail store. The intrusion will be detected resorting to an infrared sensor, installed in such a way that points towards the main entrance of the space to be protected. A switch is also installed on a window of the aforementioned space.

The automation system that constitutes the alarm is to be implemented in the Schneider PLCs available on the laboratory, model Premium TSX P57 1634M or TSX P57 2634M. This document is composed by two parts: the first describes the guidelines for the first session on the lab and the second (see annex) describes the functional specifications of all the work that will be developed in the next weeks.

# Part B - Read Keyboard

The second part of this laboratory assignment consists of the implementation of an automated solution to the identification of a key pressed, on the available keyboard with 12 keys. This device is central to the user to interact with the intrusion detection alarm system under design. The solution foreseen is the design of one or more ladder subroutine sections (optionally on structured text or instruction list), to identify the key pressed.

A suggested name for the main subroutine identifying the key pressed is SR_get_key. If no key is pressed the SR_get_key subroutine must store -1 in a memory as the result. In case a digit, '0', '1', ... , or '9', is pressed, the subroutine stores the corresponding value in the memory. In case the key '*' or the key '#' is pressed, the subroutine stores in the memory 10 or 11, respectively. A 0.05sec beep signal (buzzer sound) shall be provided as soon as a key is detected.

The SR_get_key subroutine is desired to (i) reject multiple keys pressed at the same time, (ii) recognize key release, i.e. "key up", as soon as possible and then allow immediately to read another key, and (iii) not allowing to input by mistake two or more times the same key, in other words having a repeat delay preventing immediate input of multiple times the same key. Note: a "key up" flag is useful for other parts of the program to recognize that the SR_get_key subroutine has a key that was read and is prepared to be reset and therefore input another key (notice the similarity of the SR_get_key subroutine with the timers/counters' reset and done bits).

An additional feature which will be useful for the next assignment is the possibility of buffering some keys. A subroutine named SR_get_multiple_keys must be built to hold up to 10 keys. This subroutine calls SR_get_key to get keys one by one. The SR_get_multiple_keys subroutine must have a global variable allowing to reset (flush) the buffer. The recommended implementation language for this subroutine is structured text. Note: in order to use array indexing one has to enable the option "menu Tools -> Project Settings -> variables  -> Directly represent array variables".

Report questions

Q1. Identify the PLC inputs and outputs, from/to the keyboard, to be used on the intrusion detection alarm system.

| PLC Input (chosen variable name) | PLC Identifier (physical address) |  | PLC Output (chosen variable name) | PLC Identifier (physical address) |
| --- | --- | --- | --- | --- |

(Insert as many lines as needed in the tables)

| Q2. Implement the ladder program shown in the right figure. Add a third rung to count changes in VarA. Use a wrist watch to count the time how long it takes VarA to show 1000 times the logic value "1" considering two cases: (i) VarA and VarB are memories %M1 and %M2, respectively, and (ii) VarA and VarB are the keyboard column1 and line1. As line1 is one input, it cannot be written. Thus rung2 has to be deleted from the program. Rung2 is replaced by pressing key 1 of the keyboard. Fill the next table and comment the results. |  |
| --- | --- |

Note: The physical addresses of keyboard columns and lines are usually %q0.4.x and %i0.2.y, respectively, except in the case of the DMY 28FK, where usually one has %q0.3.x and %i0.3.y.

|  | VarA | VarB | Time (sec) for 1000 pulses in VarA |
| --- | --- | --- | --- |
| Case (i) | %M1 | %M2 |  |
| Case (ii) | Kb_Column1 addr = | Kb_Line1 addr = |  |

Q3. Indicate the timer(s) and memory cell(s) used to implement SR_get_key.

| Memory or timer (chosen variable name) | PLC physical address (if specified) | Short description of the memory or timer |
| --- | --- | --- |

(Insert as many lines as needed in the table)

Q4. Describe the reading/identifying strategy to be implemented to solve the keyboard reading problem.

Q5. Upload the program proposed to the PLC and run. Comment on the results obtained.

Q6. Consult in the manuals how to use PLC system words to obtain (i) minimum and (ii) maximum scan-cycle times when the PLC is running just the SR_get_key called by one section of MAST. Indicate those times in milliseconds. Comment the next sentence: "Even in case of having access just to a low resolution time clock, e.g. a TON timer, it is possible to estimate a typical value for the scan-cycle".

Q7. Comment the following statement: "The solution to read and identify one key requires the keyboard to be scanned once, but not periodically. This is relevant in case one has relay instead of transistor based outputs."

Q8. Comment the following statement: "Not waiting for the end of reading a key may imply erroneous string readings".

# Annex - Functional Specifications of the Alarm

The Alarm has three main modes of operation, OFF, Presence Detector and Active. The three modes are selected by a three positions switch. The three modes operate as detailed next:

(Mode 1) OFF – this mode deactivates the alarm completely.

(Mode 2) PRESENCE DETECTOR – the infrared sensor is used to detect the movement on the room/space, that be signalized resorting both to a lamp and to the buzzer on the panel. The lamp should be on for 5 seconds, upon the detection of each person, and an acoustic signal with the duration of 1 second should be emitted.

(Mode 3) ACTIVE – in this mode the alarm is to be used.

Detailed specifications for mode 3, ACTIVE, are the following:

a) When requested for activation, a period of 30 seconds of inactivity is set to allow the user to abandon the space, and afterwards remains permanently activated.

b) Upon intrusion detection, by the infrared sensor or the window switch, the alarm evolves to the warning phase.

c) The alarm lights a warning on the panel and after 5 seconds the buzzer must be activated. The warning must be a periodic signal with 1 second on and 2 seconds off.

d) The alarm can be deactivated pressing the # key on the command panel.

Advanced Characteristics of the Alarm:

An advanced alternative for the alarm activation/deactivation consists on the use of a code previously set by the human owner (e.g. 9665). To implement the activation function, the following procedure must be implemented:

a) switch the alarm mode to ACTIVE.

b) introduce the activation code (e.g. 9665).

c) press #, and wait for 30 seconds to allow the user to abandon the space.

d) start the intrusion detection function, i.e. the alarm is fully operational.

To deactivate the alarm, upon intrusion detection or to allow the use of the space, the following instructions must be accomplished:

a) Introduce the secret code (the same as the activation one, e.g. 9665).

b) Press #

c) Change the alarm mode to a mode other the ACTIVE.

Special Characteristics of the Alarm:

A safer mode of operation for the intrusion detection alarm is to allow the user to change the activation/deactivation code. The code 1234 is initially used, as a factory preset. To change the code, the following operations must be done:

a) Press *, followed by the pre-programmed code.

b) Introduced the new code to be used, finished by *

In the case where a mistake occurs, press the code **** to reset the code to the factory default.

Available Material

In the laboratory there are six different working places, all with similar PLCs but different consoles. All workplaces have a PLC Schneider model P57. All of them have a power supply with 24V and/or 12V and a desktop PC, with the Unity Pro v6 development software and the PLC manuals, in PDF format.

In each workplace there will be also an alarm console with the following components:

| 12 buttons keyboard 12V buzzer 1 three positions switch 1 two positions switch 3 LEDs |  |
| --- | --- |

The solution for this automation problem must be based on the languages described on the IEC-61131-3 standard, i.e. ladder diagrams, instruction list and structured text.

2008-2010 original guide by Prof. Paulo J. Oliveira, 2011-2015 rev. by Prof. José Gaspar