|  | Industrial Processes Automation MSc in Electrical and Computer Engineering Scientific Area of Systems, Decision, and Control Winter Semester 2014/2015 |  | Group: ____ __-_____ __-_____ __-_____ __-_____ |
| --- | --- | --- | --- |

2nd Laboratory Assignment

Handling Faults in Keyboard Reading

This laboratory assignment aims at studying Discrete Event Systems (DESs) in the aspects of modeling, analysis of properties and synthesis. Synthesis will be based on a recent methodology in the framework of supervised control. This assignment further develops the previous assignment in the keyboard reading component by introducing fault handling mechanisms.

The tools to be used in this work are MATLAB and a Petri Net editor. In the last part of the work the Schneider PLCs will be used once more to validate the proposed methodologies.

# Part C - DESs Synthesis, Supervised Control

The main purpose of the second laboratory assignment is the modeling and analysis of a discrete event system, based on a Petri net. That is in close relation with the first laboratory assignment.

The main objective of this last phase, part C, is the application of the Supervisory Control theory to diagnose and isolate a failure on the system. The failure to detect occurs when the user presses two keys simultaneously. In other words, the main objective of part C consists in designing and implementing a supervisor that detects the multiple keys pressed error and resumes the normal operation of the system.

Note: see in the course webpage tools helping this part of the assignment, namely the Matlab toolbox "spnbox" which allows, for example, showing invariant places of a Petri net. The toolbox is distributed as a ZIP file named "spnbox.zip".

Q1. Design a new Petri net that, instead of reading keyboard keys, as the net proposed in part A, simply detects there are multiple keys pressed at the same time. Draw the Petri using the editor PN_Editor. Please indicate clearly the specific place of the net chosen to flag multiple keys detected. Simulate a sequence of events demonstrating the functionality of the net.

Q2. Design a supervisor controller to handle the multiple keys pressed error. Specify the linear constraint(s) that must be used. Solve the problem resorting to the place invariant methodologies studied in this course.

Q3. Design the complete resulting Petri net developed in Q1..Q2. Identify clearly the supervisor.

Q4. Discuss briefly the properties relevant for the resulting DES based on the original Petri net and on this supervisor.

| Order of importance | Property |
| --- | --- |
| 1 |  |
| 2 |  |
| 3 |  |
| 4 |  |
| 5 |  |
| 6 |  |
| 7 |  |

Q5. [Aggregation of Petri nets] Considering μp and Dp the state and incidence matrix of the Petri net designed in part A, and considering μd and Dd the state and incident matrix of the Petri net designed in part C (Q1, Q2), show that an enlarged net with state μ=[μpT μdT]T, running the two nets simultaneously, has an incidence matrix which is a function of the other two matrices, i.e. D=f(Dp, Dd). Make a Matlab program that illustrates the equivalence of using D versus using Dp and Dd.

Q6. Simulate in Matlab the complete Petri net that reads keys (developed in part A), while detecting and reacting to multiple keys pressed simultaneously (developed in part C). As a starting point, consider using the code distributed in a zip file "kb_sim_lab2.zip" (made available in the webpage of the course). Note that are empty the functions implementing the conversion of the mapping of the state to the actuation/outputs (PN_s2act.m, PN_s2yout.m), and the conversion of keyboard line signals to transitions (PN_tfire.m).

Q7. Discuss the implementation on the PLC of the supervisory control synthesized. To tackle this problem, indicate modifications to your key detection/validation program to include this new supervisor.

2008-2010 original guide by Prof. Paulo J. Oliveira, 2011-2015 rev. by Prof. José Gaspar