|  | Industrial Processes Automation MSc in Electrical and Computer Engineering Scientific Area of Systems, Decision, and Control Winter Semester 2015/2016 |  | Group: ____ __-_____ __-_____ __-_____ __-_____ |
| --- | --- | --- | --- |

1st Laboratory Assignment

Alarm System for Intrusion Detection

This work aims the implementation of an intrusion detection alarm system, in a restricted space as a single room retail store. The intrusion will be detected resorting to an infrared sensor, installed in such a way that it points towards the main entrance of the space to be protected. A switch is also installed on a window of the aforementioned space.

The automation system that constitutes the alarm is to be implemented in the Schneider PLCs available on the laboratory, model Premium TSX P57 1634M or TSX P57 2634M. This document is composed by two sections: the first describes the guidelines for the first session on the lab and the second (see annex A) describes the functional specifications of all the work that will be developed in the next weeks.

# Part A - System Components

General characteristics of the alarm:

The Alarm has three main modes of operation, namely OFF, Presence Detector and Active. The three modes are selected by a three positions switch. The general characteristics of the three modes of operation are detailed next:

(Mode 1) OFF – this mode deactivates the alarm completely.

(Mode 2) PRESENCE DETECTOR – the infrared sensor is used to detect movement in the room, and signalize it using both a lamp (LED) and the buzzer on the panel. The lamp should be on for 5 seconds, upon the detection of each person, and an acoustic signal with the duration of 1 second should be emitted.

(Mode 3) ACTIVE – in this mode the alarm is to be used.

Detailed specifications for mode 3, ACTIVE, are the following:

a) When requested for activation, a period of 30 seconds of inactivity is set to allow the user to abandon the space, and afterwards remains permanently activated.

b) Upon intrusion detection, by the infrared sensor or the window switch, the alarm evolves to the warning phase.

c) The alarm lights a warning on the panel and after 5 seconds the buzzer must be activated. The warning must be a periodic signal with 1 second ON and 2 seconds OFF.

d) The alarm can be deactivated pressing the # key on the command panel.

Implementation details

In order to implement the general characteristics of the alarm in a clear manner, it is important to separate into one sub-routine the mode 2, Presence Detector, and to another sub-routine the mode 3, Active. The indications of a detected presence and of an intrusion should also be implemented into one separate sub-routine. More sub-routines can be added in case they help making the code more clear.

In order to help the readability of the proposed solution, the identifiers used in the PLC program must have readable names. For example, instead of using "%I0.2.0" it is more readable to use "I_switch1_pos1". The following naming convention is therefore suggested: I_xyz for inputs, O_xyz for outputs, M_xyz for memory variables, TM_xyz for timers, SR_xyz for sub-routines, etc, where xyz denotes meaningful names.

Note: in this part of the work the infrared sensor is replaced by the two positions switch.

Report questions

Q1. Identify the inputs (switches) and outputs (LEDs and buzzer) of the intrusion detection alarm console and fill the next table.

| Input (chosen variable name) | PLC Identifier (physical address) |  | Output (chosen variable name) | PLC Identifier (physical address) |
| --- | --- | --- | --- | --- |

(Insert as many lines as needed in the tables)

Q2. (Hardware test program) Design and test a ladder program that verifies the proper functioning of the output devices available in the alarm console. More precisely, the program must beep the buzzer 0.5 seconds and turn ON the three lamps (LEDs), one after the other, 0.5 seconds each. (Add space here to answer the question.)

Q3. (Keypad test) Use a multimeter to label the input and output wires of a 4x3 keypad (see appendix). Use the following nomenclature: number 1,2,3 the 3 columns and name  a,b,c,d the 4 lines. Write and run a Ladder program to test the keypad with the following specification:

if %MW0 is 0 then reset variables %M1 till %M4, and set %MW0 equal to 1;

if %MW0 is 1 then activate keypad column 1;

if %MW0 is 1 and the keypad line 1 is active then set %M1 and set %MW1 equal to 2;

if %MW0 is 2 then activate keypad column 2;

if %MW0 is 2 and the keypad line 2 is active then set %M2 and set %MW1 equal to 3;

(...) complete this program to test also column 3 and lines 3 and 4 of the keypad.

(Add space here to answer the question.)

Q4. (Multiple accesses to the buzzer) Create a ladder sub-routine where the buzzer can be turned ON by one of two alternative memory variables (BOOL), and can be unconditionally turned OFF by another memory variable (BOOL). Create a third way of actuating the buzzer, also commanded by a memory variable (BOOL), where the buzzer sounds with a periodic signal, 1 second ON and 2 seconds OFF. (Add space here to answer the question.)

Q5. Consider the alarm application in this and the next questions. List and describe the timers that will be used, their function, and the delay times to be selected for each one.

| Timer name and/or physical address | Operation mode | Time Delay | Short description of the usage of the timer |
| --- | --- | --- | --- |

Q6. List the sub-routines to implement in the alarm application. In particular indicate the routine names and list variables shared with other routines or sub-routines. Classify the listed variables as "input", "output" or "input and output".

Q6. Design one or more ladder sections to solve the aforementioned automation problem. (Add an extra sheet of paper to answer this question).

Q7. Upload the program to the PLC and execute it. Comment how it runs.

# Annex A - Functional Specifications of the Alarm

The Alarm has three main modes of operation, OFF, Presence Detector and Active. The three modes are selected by a three positions switch. The three modes operate as detailed next:

(Mode 1) OFF – this mode deactivates the alarm completely.

(Mode 2) PRESENCE DETECTOR – the infrared sensor is used to detect the movement on the room/space, that be signalized resorting both to a lamp and to the buzzer on the panel. The lamp should be on for 5 seconds, upon the detection of each person, and an acoustic signal with the duration of 1 second should be emitted.

(Mode 3) ACTIVE – in this mode the alarm is to be used.

Detailed specifications for mode 3, ACTIVE, are the following:

a) When requested for activation, a period of 30 seconds of inactivity is set to allow the user to abandon the space, and afterwards remains permanently activated.

b) Upon intrusion detection, by the infrared sensor or the window switch, the alarm evolves to the warning phase.

c) The alarm lights a warning on the panel and after 5 seconds the buzzer must be activated. The warning must be a periodic signal with 1 second on and 2 seconds off.

d) The alarm can be deactivated pressing the # key on the command panel.

Advanced Characteristics of the Alarm:

An advanced alternative for the alarm activation/deactivation consists on the use of a code previously set by the human owner (e.g. 9665). To implement the activation function, the following procedure must be implemented:

a) switch the alarm mode to ACTIVE.

b) introduce the activation code (e.g. 9665).

c) press #, and wait for 30 seconds to allow the user to abandon the space.

d) start the intrusion detection function, i.e. the alarm is fully operational.

To deactivate the alarm, upon intrusion detection or to allow the use of the space, the following instructions must be accomplished:

a) Introduce the secret code (the same as the activation one, e.g. 9665).

b) Press #

c) Change the alarm mode to a mode other the ACTIVE.

Special Characteristics of the Alarm:

A safer mode of operation for the intrusion detection alarm is to allow the user to change the activation/deactivation code. The code 1234 is initially used, as a factory preset. To change the code, the following operations must be done:

a) Press *, followed by the pre-programmed code.

b) Introduced the new code to be used, finished by *

In the case where a mistake occurs, press the code **** to reset the code to the factory default.

Available Material

In the laboratory there are six different working places, all with similar PLCs but different consoles. All workplaces have a PLC Schneider model P57. All of them have a power supply with 24V and/or 12V and a desktop PC, with the Unity Pro v6 development software and the PLC manuals, in PDF format.

In each workplace there will be also an alarm console with the following components:

| 12 buttons keyboard 12V buzzer 1 three positions switch 1 two positions switch 3 LEDs |  |
| --- | --- |

The solution for this automation problem must be based on the languages described on the IEC-61131-3 standard, i.e. ladder diagrams, instruction list and structured text.

# Annex B - 4x3 Keypad

The keypads that are used in the laboratory have 12 keys arranged in a 4x3 matrix (see the next figure). The terminology to be used in the laboratory is the following: the three columns are named by the digits 1,2,3 and the four lines by the letters a,b,c,d.

The next image shows the inside of a keypad. Notice that keypads can be made simply of contacts which are short circuited when someone presses a button. For example pressing key "eight" shorts the circuit of column 2 with line c. Most 4x3 keypads have 4+3 wires. Some, as the one shown in the figure bellow, can have also an 8th wire which will not be used in the project.

In this picture the pads with holes for wiring the keypad to a terminal (or a PLC) are marked, from left-to-right, 3,2,1,a,b,c,d. This sorting is not standard among manufacturers. In most cases columns and lines are mixed and unsorted as the base printed circuit boards are simpler. Every 4x3 keypad must be tested to identify the pads corresponding to the 3 columns and the 4 lines.

A possible algorithm for identifying the 3 pads that can act the columns and the 4 pads that can read the lines is the following:

- Choose 2 of the 7 pads of the keyboard and use the multimeter (ohmmeter) to verify that after pressing all keys the 2 pads (i) can or (ii) cannot be short circuited.

- If case (i), i.e. the two pads can be short circuited by a pressed key, then press another key in the same column and change one of the two pads to the other 5 positions. If the novel key is not identified, then return the changed pad to its original location and do the same 5 tests with the other pad. This procedure allows naming a pad as column 1, 2 or 3.

- If case (ii), then the two chosen pads connect to two lines or to two columns. Need to reselect one pad of the selected pair. Repeat this until a pair of pads is in accordance with case (i).

- After identifying the pad defining one column, it is easy to identify 4 pads corresponding to the 4 lines, as pressing keys in the right column must always do a short circuit to another pad.

- Having identified 1+4 pads, the remaining two pads are connected to the two other columns. Pressing a key in one column or the other makes a short circuit to one of the four lines.

2008-2010 original guide by Prof. Paulo J. Oliveira, 2011-2016 rev. by Prof. José Gaspar