|  | Industrial Processes Automation MSc in Electrical and Computer Engineering Scientific Area of Systems, Decision, and Control Winter Semester 2018/2019 |  | Group: ____ __-_____ __-_____ __-_____ __-_____ |
| --- | --- | --- | --- |

1st Laboratory Assignment

Alarm System for Intrusion Detection

This work aims the implementation of an intrusion detection alarm system, in a restricted space as a single room retail store. The intrusion will be detected resorting to an infrared sensor, installed in such a way that points towards the main entrance of the space to be protected. A switch is also installed on a window of the space.

The automation system that constitutes the alarm is to be implemented in the Schneider PLCs available on the laboratory, model Premium TSX P57 1634M or TSX P57 2634M. This document is composed by two parts: the first describes the guidelines for the session on the lab and the second (see annex) describes the functional specifications of all the work that will be developed in the next weeks.

# Part C - Integration

In this part of the work the 12-keys keyboard available in the console is combined with the switches to build the primary interaction device with the alarm system under design. Sequences of keys must be validated resorting to the subroutines implemented in the previous part of this work. The integration of the complete system is suggested to be done in GRAFCET / Sequential Function Chart (SFC).

Implementation guidelines

The basic presence detection and active alarm system already developed is supposed to be running continuously. Note however that the function of selecting the operating mode (presence detection / active alarm) cannot be based just on switches, must be moved to an upper integration level (SFC).

Keyboard reading is also a function assumed to be running continuously. The upper integration level can validate codes saved in a buffer. Keyboard input is cleared if a timeout occurs or any other condition recommends flushing keys already read.

It is recommended to allow testing the hardware before entering into the active alarm mode. This option can be made available before entering the activation code by pressing the # key of the keyboard. After the hardware test, and in case the active alarm mode switch is ON, one can enter the activation code and therefore enter the active alarm mode.

In order to facilitate the interaction, any return to the OFF mode must be marked with 3-short-beeps done with the buzzer. A suggested timing is 0.02sec for each beep and a pause of 0.1sec in between.

Summary of the assignment

Please develop, implement, and test one or more sections in GRAFCET / SFC that, according with the interaction with the user, activates a presence detection mode, validates sequences of keys to activate the alarm, to change the code, restart the code to factory settings, as well as other specifications in the annex. Please write the receptivity functions in Ladder, Instruction List, or Structured Text.

Report questions

Note: this guide is distributed in an editable form so that it can be filled and then submitted online.

Q1. Represent as a finite state machine the upper integration level of the complete system. Clearly identify the OFF mode, the presence detection mode, the active alarm mode, and the other states necessary for the complete system.

Q2. Design a GRAFCET / SFC program representing the upper integration level. Please use macro steps to help making simple the upper integration level. Describe the implemented receptivity sections. Please write comments within the code to improve readability.

Q3. Describe the methodology used to validate sequences of keys of a pre-specified length. Discuss how to proceed if the input fails. In particular, identify the auxiliary variables (memory) where the code sequence information is stored and the timers used for implementing the sequence validation procedure.

Q4. Upload the program to the PLC, execute it and comment the results. Use logging-methodologies (see part A question 9) and create plots that illustrate the functioning of the system.

Q5. Indicate the minimum time pressing keys till successfully deactivating the active alarm mode. Indicate also the maximum time, if applicable.

Q6. Draw a hardware diagram that allows testing automatically your solution by using hardware as 
http://users.isr.ist.utl.pt/~jag/course_utils/plc_interf/plc_flat_cable_interf.html

Q7. Discuss possible future improvements to the alarm system. In particular try to identify aspects the user of the system would like to see ameliorated.

# Annex A - Functional Specifications of the Alarm

(please do NOT include this annex in your report)

The Alarm has three main modes of operation, OFF, Presence Detector and Active. The three modes are selected by a three positions switch. The three modes operate as detailed next:

(Mode 1) OFF – this mode deactivates the alarm completely.

(Mode 2) PRESENCE DETECTOR – the infrared sensor is used to detect the movement on the room/space, that be signalized resorting both to a lamp and to the buzzer on the panel. The lamp should be on for 5 seconds, upon the detection of each person, and an acoustic signal with the duration of 1 second should be emitted.

(Mode 3) ACTIVE – in this mode the alarm is to be used.

Detailed specifications for mode 3, ACTIVE, are the following:

a) When requested for activation, a period of 30 seconds of inactivity is set to allow the user to abandon the space, and afterwards remains permanently activated.

b) Upon intrusion detection, by the infrared sensor or the window switch, the alarm evolves to the warning phase.

c) The alarm lights a warning on the panel and after 5 seconds the buzzer must be activated. The warning must be a periodic signal with 1 second on and 2 seconds off.

d) The alarm can be deactivated pressing the # key on the command panel.

Advanced Characteristics of the Alarm:

An advanced alternative for the alarm activation/deactivation consists on the use of a code previously set by the human owner (e.g. 9887). To implement the activation function, the following procedure must be implemented:

a) switch the alarm mode to ACTIVE.

b) introduce the activation code (e.g. 9887).

c) press #, and wait for 30 seconds to allow the user to abandon the space.

d) start the intrusion detection function, i.e. the alarm is fully operational.

To deactivate the alarm, upon intrusion detection or to allow the use of the space, the following instructions must be accomplished:

a) Introduce the secret code (the same as the activation one, e.g. 9887).

b) Press #

c) Change the alarm mode to a mode other the ACTIVE.

Special Characteristics of the Alarm:

A safer mode of operation for the intrusion detection alarm is to allow the user to change the activation/deactivation code. The code 1234 is initially used, as a factory preset. To change the code, the following operations must be done:

a) Press *, followed by the pre-programmed code.

b) Introduced the new code to be used, finished by *

In the case where a mistake occurs, press the code **** to reset the code to the factory default.

Available Material

In the laboratory there are six different working places, all with similar PLCs but different consoles. All workplaces have a PLC Schneider model P57. All of them have a power supply with 24V and/or 12V and a desktop PC, with the Unity Pro v6 development software and the PLC manuals, in PDF format.

In each workplace there will be also an alarm console with the following components:

| 1 three positions switch 1 two positions switch 3 LEDs 1 keyboard (4x3 buttons) 1 buzzer (12V) |  |
| --- | --- |

The solution for this automation problem must be based on the languages described on the IEC-61131-3 standard, i.e. ladder diagrams, instruction list and structured text.

Original guide by Prof. Paulo J. Oliveira. 2019 revision by Prof. José Gaspar.